ABSTRACT

The transfer of energy from a source to a receiver has traditionally necessitated the use of a physical connection. Indeed, electrical grids and power outlets span nearly the entire globe and deliver power to billions of people worldwide. Recently, there has been much interest into the area of wireless power transfer (WPT), that is, the transmission of power without the need for a physical connection. WPT is an extremely useful technology that has numerous applications and benefits. Cell phones, laptops and other mobile devices could function without ever having to be plugged in, cars could drive on highways burning no fossil fuels; wireless power even has the potential to solve much of the renewable energy issues we face.

CHAPTER 1:

INTRODUCTION:

Wireless power transfer (WPT) or wireless energy transmission is the transmission of electrical power from a power source to a consuming device without using discrete manmade conductors. It is a generic term that refers to a number of different power transmission technologies that use time-varying electromagnetic fields. Wireless transmission is useful to power electrical devices in cases where interconnecting wires are inconvenient, hazardous, or are not possible. In wireless power transfer, a transmitter device connected to a power source, such as the mains power line, transmits power by electromagnetic fields across an intervening space to one or more receiver devices, where it is converted back to electric power and utilized.

Wireless power techniques fall into two categories, non-radiative and radiative. In near-field or non-radiativetechniques, power is transferred over short distances by magnetic fields using inductive coupling between coils of wire or in a few devices by electric fields using capacitive coupling between electrodes.[5][8] Applications of this type are electric toothbrush chargers, RFID tags, smartcards, and chargers for implantable medical devices like artificial cardiac pacemakers, and inductive powering or charging of electric vehicles like trains or buses. A current focus is to develop wireless systems to charge mobile and handheld computing devices such as cellphones, digital music players and portable computers without being tethered to a wall plug.

## Overview

Generic block diagram of a wireless power system

"Wireless power transmission" is a collective term that refers to a number of different technologies for transmitting power by means of time-varying electromagnetic fields. The technologies, listed in the table below, differ in the distance over which they can transmit power efficiently, whether the transmitter must be aimed (directed) at the receiver, and in the type of electromagnetic energy they use: time varying electric fields, magnetic fields, radio waves, microwaves, or infrared or visible light waves.

In general a wireless power system consists of a "transmitter" device connected to a source of power such as mains power lines, which converts the power to a time-varying electromagnetic field, and one or more "receiver" devices which receive the power and convert it back to DC or AC electric power which is consumed by an electrical load. In the transmitter the input power is converted to an oscillating electromagnetic field by some type of "antenna" device. The word "antenna" is used loosely here; it may be a coil of wire which generates a magnetic field, a metal plate which generates an electric field, an antenna which radiates radio waves, or a laser which generates light. A similar antenna or coupling device in the receiver converts the oscillating fields to an electric current. An important parameter which determines the type of waves is the frequency f in hertz of the oscillations. The frequency determines the wavelength λ = c/f of the waves which carry the energy across the gap, where c is the velocity of light.

Wireless power uses the same fields and waves as wireless communication devices like radio, another familiar technology which involves power transmitted without wires by electromagnetic fields, used in cell phones, radio and television broadcasting, and Wi-Fi. In radio communication the goal is the transmission of information, so the amount of power reaching the receiver is unimportant as long as it is enough that the signal to noise ratio is high enough that the information can be received intelligibly. In wireless communication technologies generally only tiny amounts of power reach the receiver. By contrast, in wireless power, the amount of power received is the important thing, so the efficiency (fraction of transmitted power that is received) is the more significant parameter. For this reason wireless power technologies are more limited by distance than wireless communication technologies

#### Resonant inductive coupling

Resonant inductive coupling (electrodynamic coupling, evanescent wave coupling or strongly coupled magnetic resonance) is a form of inductive coupling in which power is transferred by magnetic fields (Bbetween two resonant circuits (tuned circuits), one in the transmitter and one in the receiver Each resonant circuit consists of a coil of wire connected to a capacitor, or a self-resonant coil or other resonator with internal capacitance. The two are tuned to resonate at the same resonant frequency. The resonance between the coils can greatly increase coupling and power transfer, analogously to the way a vibrating tuning fork can induce sympathetic vibration in a distant fork tuned to the same pitch. Nikola Tesla first discovered resonant coupling during his pioneering experiments in wireless power transfer around the turn of the 20th century,[37][38][39] but the possibilities of using resonant coupling to increase transmission range has only recently been explored.[40] In 2007 a team led by Marin Soljačić at MIT used two coupled tuned circuits each made of a 25 cm self-resonant coil of wire at 10 MHz to achieve the transmission of 60 W of power over a distance of 2 meters (6.6 ft) (8 times the coil diameter) at around 40% efficiency.

The concept behind resonant inductive coupling is that high Q factor resonators exchange energy at a much higher rate than they lose energy due to internal damping. Therefore, by using resonance, the same amount of power can be transferred at greater distances, using the much weaker magnetic fields out in the peripheral regions ("tails") of the near fields (these are sometimes called evanescent fields). Resonant inductive coupling can achieve high efficiency at ranges of 4 to 10 times the coil diameter (Dant). This is called "mid-range" transfer, in contrast to the "short range" of non-resonant inductive transfer, which can achieve similar efficiencies only when the coils are adjacent. Another advantage is that resonant circuits interact with each other so much more strongly than they do with non-resonant objects that power losses due to absorption in stray nearby objects are negligible. A drawback of resonant coupling is that at close ranges when the two resonant circuits are tightly coupled, the resonant frequency of the system is no longer constant but "splits" into two resonant peaks, so the maximum power transfer no longer occurs at the original resonant frequency and the oscillator frequency must be tuned to the new resonance peak.

Resonant technology is currently being widely incorporated in modern inductive wireless power systems.[32] One of the possibilities envisioned for this technology is area wireless power coverage. A coil in the wall or ceiling of a room might be able to wirelessly power lights and mobile devices anywhere in the room, with reasonable efficiency. An environmental and economic benefit of wirelessly powering small devices such as clocks, radios, music players and remote controls is that it could drastically reduce the 6 billion batteries disposed of each year, a large source of toxic waste and groundwater contamination.

## Benefits of Wireless Power

- Reduce costs associated with maintaining direct connectors

- Greater convenience for ​the charging of everyday electronic devices

- Safe​ power transfer to applications that need to remain sterile or hermetically sealed

- Electronics can be fully enclosed, reducing the risk of corrosion due to elements such as oxygen and water.

- ​Robust and consistent power delivery to rotating, highly mobile industrial equipment​

- ​Delivers reliable power transfer to mission critical systems in wet, dirty and moving environments.

VOLTAGE CONVERTER

A voltage converter changes the voltage of an electrical power source and is usually combined with other components to create a power supply. The term "voltage converter" is sometimes used as a generic term for a power supply. A voltage converter or a power supply may be called a "transformer" even if it does not contain an actual transformer as the term is used in electronics.

A common use of the term voltage converter is for a device that allows appliances made for the voltage of one geographical region to operate from the power outlets of another area.[1] Most single phase alternating current electrical outlets in the world supply power at 210 to 230 volts or at 100 to 125 volts. Converters usually can only double or halve the voltage, but some can do both.

Often these are sold with plug end adaptors that provide physical compatibility of normally-incompatible plugs and sockets.

There are a number of methods for converting voltage. For Alternating current (AC) a transformer can be used to decrease or increase the voltage. The common power supplies for small electronics usually have a transformer that drops the voltage down to relatively small voltage ranging from 4.5 to 12 volts, a full-wave rectifier to convert the AC to a pulsed Direct Current and some additional components to flatten the wave. Some devices have only a transformer in the external power supply with any rectifier or additional power conditioning provided inside the appliance.

Voltage converters sold for use in cars with 12 volt Direct Current outlets almost always have no transformer and instead use a Zener diode to drop the DC voltage with a relatively large power loss as heat. Converting a low DC voltage to a high DC voltage requires conversion to AC, the use of a transformer to increase the voltage and then a rectifier and conditioning circuit to convert it back to DC. Going through an AC step can also be used to drop voltage. For example one might use an inverter to produce 110 AC current from a car's electrical system and then use the conventional power supply that came with the device.

There are two types of voltage converters, step up and step down. Step up converts from 110 volts to 230 volts and step down will convert from 230 volts to 110 volts. Most voltage converters convert both ways.

DC to DC converter

A DC to DC converter is a circuit which converts a source of direct current (DC) from one voltage level to another. It is a class of power converter. DC to DC converters are important in portable electronic devices such as cellular phones and laptop computers, which are supplied with power from batteries primarily. Such electronic devices often contain several sub-circuits, each with its own voltage level requirement different than that supplied by the battery or an external supply (sometimes higher or lower than the supply voltage, and possibly even negative voltage). Additionally, the battery voltage declines as its stored power is drained. Switched DC to DC converters offer a method to increase voltage from a partially lowered battery voltage thereby saving space instead of using multiple batteries to accomplish the same thing.

Conversion methods

Linear regulators can output a lower, but not a higher, voltage from the input. They are very inefficient if the voltage drop is large and the current high as they dissipate as heat power equal to the product of the output current and the voltage drop; consequently they are not normally used for large-drop high-current applications.

The inefficiency wastes power and requires higher-rated, and consequently more expensive and larger, components. The heat dissipated by high-power supplies is a problem in itself as it must be removed from the circuitry to prevent unacceptable temperature rises.

They are practical if the current is low, the power dissipated being small, although it may still be a large fraction of the total power consumed. They are often used as part of a simple regulated power supply for higher currents: a transformer generates a voltage which, when rectified, is a little higher than that needed to bias the linear regulator. The linear regulator drops the excess voltage, reducing hum-generating ripple current and providing a constant output voltage independent of normal fluctuations of the unregulated input voltage from the transformer / bridge rectifier circuit and of the load current.

Linear regulators are inexpensive, reliable if good heat sinking is used and much simpler than switching regulators. As part of a power supply they may require a transformer, which is larger for a given power level than that required by a switch-mode power supply. Linear regulators can provide a very low-noise output voltage, and are very suitable for powering noise-sensitive low-power analog and radio frequency circuits. A popular design approach is to use an LDO, Low Drop-out Regulator that provides a local "point of load" DC supply to a low power circuit.

Switched-mode conversion

Electronic switch-mode DC to DC converters convert one DC voltage level to another, by storing the input energy temporarily and then releasing that energy to the output at a different voltage. The storage may be in either magnetic field storage components (inductors, transformers) or electric field storage components (capacitors). This conversion method is more power efficient (often 75% to 98%) than linear voltage regulation (which dissipates unwanted power as heat). This efficiency is beneficial to increasing the running time of battery operated devices. The efficiency has increased in since the late 1980's due to the use of power FETs, which are able to switch at high frequency more efficiently than power bipolar transistors, which have more switching losses and require a more complex drive circuit. Another important innovation in DC-DC converters is the use of synchronous switching which replaces the flywheel diode with a power FET with low "On" resistance, there by reducing switching losses.

Drawbacks of switching converters include complexity, electronic noise (EMI / RFI) and to some extent cost, although this has come down with advances in chip design.

DC to DC converters are now available as integrated circuits needing minimal additional components. DC to DC converters are also available as a complete hybrid circuit component, ready for use within an electronic assembly.

Magnetic

In these DC to DC converters, energy is periodically stored into and released from a magnetic field in an inductor or a transformer, typically in the range from 300 kHz to 500 MHz. By adjusting the duty cycle of the charging voltage, that is the ratio of on/off time, the amount of power transferred can be controlled. Usually, this is done to control the output voltage, though it could be done to control the input current, the output current, or maintaining a constant power. Transformer based converters may provide isolation between the input and the output. In general, the term "DC to DC converter" refers to one of these switching converters. These circuits are the heart of a switched-mode power supply. Many topologies exist. This table shows the most common.

In addition, each topology may be:

- Hard switched - transistors switch quickly while exposed to both full voltage and full current

- Resonant - an LC circuit shapes the voltage across the transistor and current through it so that the transistor switches when either the voltage or the current is zero

Magnetic DC to DC converters may be operated in two modes, according to the current in its main magnetic component (inductor or transformer):

- Continuous - the current fluctuates but never goes down to zero

- Discontinuous - the current fluctuates during the cycle, going down to zero at or before the end of each cycle

A converter may be designed to operate in Continuous mode at high power, and in Discontinuous mode at low power.,

The Half Bridge and Flyback topologies are similar in that energy stored in the magnetic core needs to be dissipated so that the core does not saturate. Power transmission in a flyback circuit is limited by the amount of energy that can be stored in the core, while forward circuits are usually limited by the I/V characteristics of the switches.

Mosfet switches can tolerate simultaneous full current and voltage (although thermal stress and electromigration can shorten the MTBF), bipolar switches generally don't so require the use of a snubber (or two).

Capacitive

Switched capacitor converters rely on alternately connecting capacitors to the input and output in differing topologies. For example, a switched-capacitor reducing converter might charge two capacitors in series and then discharge them in parallel. This would produce an output voltage of half the input voltage, but at twice the current (minus various inefficiencies). Because they operate on discrete quantities of charge, these are also sometimes referred to as charge pump converters. They are typically used in applications requiring relatively small amounts of current, as at higher current loads the increased efficiency and smaller size of switch-mode converters makes them a better choice.[citation needed] They are also used at extremely high voltages, as magnetics would break down at such voltages.

Electrochemical

A further means of DC to DC conversion in the kW to many MW range is presented by using redox flow batteries such as the vanadium redox battery, although this technique has not been applied commercially to date.

Step-down DC/DC converter input ripple and noise

This application note discusses the proper methods for measuring input and output noise of a high frequency DC/DC converter. Without proper measurement techniques, the high frequency noise component can easily be overestimated. When necessary, simple methods are available to reduce input noise and the impact it may have on other devices.

When a step-down converter shares a common input voltage with other devices, the input noise generated by the step-down converter may be sufficient to cause interference with the other devices. Simple filter methods can be used to reduce the input noise, and simple models can be developed characterise the circuit. Two circuit models&#mdash;one for the fundamental switching frequency ripple component and one for the high frequency noise&#mdash; simplify the analyses. In most cases, a ceramic bypass capacitor placed close to devices that share the common input voltage is sufficient. A ferrite bead placed between the converter input and other devices powered from the same source attenuates the converter input noise.

Class D:

The purpose of a radio frequency power amplifier is to deliver, with reasonable efficiency, a sinusoidal signal having low harmonics and other spurious components. Rf power amplifiers are classified by the operation of their output transistors. Classes A to C uses transistors in the active region, so they are generally unsuitable for use in power converters, where efficiency is far more important than signal purity.

To increase efficiency, the transistors may instead be operated as switches. Class D is a two-switch topology in which the switches conduct on alternate half cycles, each with a conduction angle approaching 180°. A series resonant tank converts the square voltage waveform into a sinusoidal load current.

Literature review:

- Nabae, I. Takahashi, and H. Akagi, “A new neutral-point-clamped PWM inverter,” IEEE Trans. Ind. Appl., vol. IA-17, no. 5, pp. 518–523, Sep. 1981.

A new neutral-point-clamped pulsewidth modulation (PWM) inverter composed of main switching devices which operate as switches for PWM and auxiliary switching devices to clamp the output terminal potential to the neutral point potential has been developed. This inverter output contains less harmonic content as compared with that of a conventional type. Two inverters are compared analytically and experimentally. In addition, a new PWM technique suitable for an ac drive system is applied to this inverter. The neutral-point-clamped PWM inverter adopting the new PWM technique shows an excellent drive system efficiency, including motor efficiency, and is appropriate for a wide-range variable-speed drive system.

- F. Zhang, F. Z. Peng, and Z. Qian, “Study of the multilevel converters in DC-DC applications,” in Proc. IEEE 35th Annu. Power Electron. Spec. Conf., Jun. 2004, pp. 1702–1706, vol. 2.

Multilevel converters have been demonstrated to have many advantages such as low harmonic, low voltage stress, and high power capability. However, most of the researches are focused on high power AC-DC and DC-AC applications. This paper presents several multilevel DC-DC converters, which can be used in automotive applications as well as in high power applications. Based on the diode-clamp, flying-capacitor, and cascaded multilevel inverters, their correlative DC-DC converters are derived.

- D. Ma et al., “Single-inductor multiple-output switching converters with time-multiplexing control in discontinuous conduction mode,” IEEE J. Solid-State Circuits, vol. 38, no. 1, pp. 89–100, Jan. 2003.

An integrated single-inductor dual-output boost converter is presented. This converter adopts time-multiplexing control in providing two independent supply voltages (3.0 and 3.6 V) using only one 1-/spl mu/H off-chip inductor and a single control loop. This converter is analyzed and compared with existing counterparts in the aspects of integration, architecture, control scheme, and system stability. Implementation of the power stage, the controller, and the peripheral functional blocks is discussed.

Chapter 2

Conventional system:

The conventional circuit consists of input DC supply full bridge rectifier then the transmitting coil and receiving coil, then the full bridge rectifier circuit.

DISADVANTAGES

  - No of switches high

  - Ripple factor high due to diode rectifier

  - Single output

  - Low efficiency.

Chapter 3

Proposed system:

r

DESCRIPTION OF THE PROPOSED WIRELESS POWER TRANSMISSION SYSTEM

High Step-up DC-DC Converte     Class E amplifier stage

C1	D3

D2

L1

S1

C5        L2     C6

C7

C3

PV

Lm

D1

S2

C4

C0      R0

C2

The fig 2 shows the two main stages of the proposed system. The first stage is the high step-up dc-dc converter which converts the low input voltage from the PV Cell to a higher value. The step-up converter has following advantages:

  - The converter has a high step-up conversion ratio because of the connection of the coupled inductors, diodes and the capacitors.

  - It has very high efficiency and lower stress on the switches as the leakage inductor energy can be recycled.

It consists of a coupled inductor T1 with the switch S1.The primary side winding N1 of a coupled inductor T1 is identical to the input inductor of the traditional boost converter, and diode D1, capacitor C1 receives leakage inductor energy from N1. The secondary side winding N2 of coupled inductor T1 is connected with another pair of diode D2 and capacitors C2, which are in series with N1 in order to increase the boost voltage. The rectifier diode D3 is connected to output capacitor C3.

The second stage is the class E amplifier which receives the dc input from the high step-up converter and converts to high frequency ac. The class E amplifier is a high efficient switch mode resonant converter. The high efficiency results from the reduced power losses in the transistor. The higher efficiency of the switch can be achieved by:

- Using the transistor as a switch to reduce the power

- Reducing the switching losses which result from finite transition time between ON and OFF states of the transistor.

The Class E amplifier consists of a RF choke L1 and a parallel-series resonator circuit consisting of C4, C5 and L2.

The output of the class E power amplifier is connected to the tank circuit formed by C6 and the transmitting coil as shown  in the fig 2.

The receiver consists of a tank circuit formed by capacitor C7 and the receiving coil and a simple full bridge diode rectifier to convert the ac power transmitted from the transmitter coil to dc and a filter C0 is used to reduce the harmonics and then given to the load R0. The power gets transferred resonant frequency is achieved between transmitter and receiver pair.

MODES OF OPERATION OF THE PROPOSED WIRELESS POWER TRANSMISSION SYSTEM

There are five modes of operation for high step-up dc-dc converter and class E amplifier has only two modes which will be discussed separately.

- Modes of Operation of High Step-Up DC-DC Converter:

Mode I (t0 – t1): Fig 3 shows the mode I operation of the step-up converter. When the switch S1 is closed, the capacitor C2  gets completely charged  by the  magnetizing inductor  Lm.

L4

The magnetizing inductor current ILm  decreases as the input voltage Vin crosses the magnetizing inductor Lm and the leakage inductor Lk1. Lm still continues to transfer energy to  the capacitor C2 but this energy is decreasing. The current through the diode D2 and the capacitor C2 are also decreasing. The secondary leakage current iLk2 is also decreasing with a slope of iLm/n. This mode ends when increasing iLk1 is equal to the decreasing iLm  at t=t1.

Mode II (t1 – t2): Fig 4 shows the mode II operation of the step-up converter. During this mode, the input source voltage Vin gets series connected with N2, C1 and C2 which charge the output capacitor C3. The currents iLm, iLk1 and id3 increases as Vin crosses Lk1, Lm and N1. Lm and Lk1 stores energy from Vin also C1 and C2 discharge their energy toC3. Hence id3 and the discharging currents ic1 and ic2 also increase. The switch is turned off at t=t2  and this mode ends.

Mode III (t2 – t3): Fig 5 shows the  mode III operation of the step-up converter. During this mode the secondary leakage inductor Lk2 keeps charging the output capacitor C3 when the switch is turned off at t=t2. Diodes D1 and D3 will be conducting. The stored energy in Lk1 flows through D1 to charge the capacitor C1. Also, the stored energy in the leakage inductor Lk2 is in series with C2 to charge the output capacitor C3. Since the inductances of Lk1 and Lk2 are very small compared to Lm, iLk2 decreases rapidly but iLm increases as the magnetizing inductor Lm receives energy from Lk1. This mode ends when iLk2  decreases and reaches zero at t = t3.

Mode IV (t3 – t4): Fig 6 shows the  mode IV operation of the step-up converter. The magnetizing inductor Lm discharges its energy to C1 and C2. Diodes D1 and D2 are conducting in this mode. The currents i0 and iD1 are decreases continuously  as the leakage energy charge the capacitor C1  through  the diode D1. The  magnetizing inductor  Lm  discharges its energy to charge the capacitor C2 through T1 and D2. The  energy stored in C3 is continuously discharged to the load R. These energy transfers decreases the currents iLk1 and iLm but increases the current iLk2.This mode ends when iLk1 reaches zero at t=t4.

Applications:

- Electric vehicle applications

- Electronic portable devices

Cell phones,

laptops, tablets,

smart watches

- Aerial Vehicles and Solar Power Satellites

CHAPTER 5

SIMULATION

Introduction:

MATLAB is a high-performance language for technical computing. It integrates computation, visualization, and programming in an easy-to-use environment where problems and solutions are expressed in familiar mathematical notation.Typical uses include

- Math and computation

- Algorithm  development

- Data acquisition

- Modeling, simulation, and prototyping

- Data analysis, exploration, and visualization

- Scientific and engineering graphics

- Application development, including graphical user interface building

MATLAB is an interactive system whose basic data element is an array that does not require dimensioning. This allows you to solve many technical computing problems, especially those with matrix and vector formulations, in a fraction of the time it would take to write a program in a scalar non interactive language such as C or Fortran.

The name MATLAB stands for matrix laboratory. MATLAB was originally written to provide easy access to matrix software developed by the LINPACK and EISPACK projects. Today, MATLAB engines incorporate the LAPACK and BLAS libraries, embedding the state of the art in software for matrix computation

MATLAB has evolved over a period of years with input from many users. In university environments, it is the standard instructional tool for introductory and advanced courses in mathematics, engineering, and science. In industry, MATLAB is the tool of choice for high-productivity research, development, and analysis.

MATLAB features a family of add-on application-specific solutions called toolboxes .Very important to most users of MATLAB, toolboxes allow you to learn and apply specialized technology. Toolboxes are comprehensive collections of MATLAB functions (M-files) that extend the MATLAB environment to solve particular classes of problems. Areas in which toolboxes are available include signal processing, control systems, neural networks, fuzzy logic, wavelets, simulation, and many others.

SimPowerSystems:

SimPowerSystems and other products of the Physical Modeling product family work together with Simulink® to model electrical, mechanical, and control systems. SimPower Systems operates in the Simulink environment. Therefore, before starting this user's guide, you should be familiar with Simulink. For help with Simulink, see the Simulink documentation. Or, if you apply Simulink to signal processing and communications tasks (as opposed to control system design tasks), see the Signal Processing Blockset documentation.

The Role of Simulation in Design:

Electrical power systems are combinations of electrical circuits and electromechanical devices like motors and generators. Engineers working in this discipline are constantly improving the performance of the systems. Requirements for drastically increased efficiency have forced power system designers to use power electronic devices and sophisticated control system concepts that tax traditional analysis tools and techniques. Further complicating the analyst's role is the fact that the system is often so nonlinear that the only way to understand it is through simulation .Land-based power generation from hydroelectric, steam, or other devices is not the only use of power systems. A common attribute of these systems is their use of power electronics and control systems to achieve their performance objectives. SimPower Systems is a modern design tool that allows scientists and engineers to rapidly and easily build models that simulate power systems. SimPower Systems uses the Simulink environment, allowing you to build a model using simple click and drag procedures. Not only can you draw the circuit topology rapidly, but your analysis of the circuit can include its interactions with mechanical, thermal, control, and other disciplines. This is possible because all the electrical parts of the simulation interact with the extensive Simulink modeling library. Since Simulink uses MATLAB® as its computational engine, designers can also use MATLAB toolboxes and Simulink blocksets. SimPower Systems and Sim Mechanics share a special Physical Modeling block and connection line interface.

SimPower Systems Libraries

The libraries contain models of typical power equipment such as transformers, lines, machines, and power electronics. These models are proven ones coming from textbooks, and their validity is based on the experience of the Power Systems Testing and Simulation Laboratory of Hydro-Québec, a large North American utility located in Canada, and also on the experience of Evolve de Technologies superior and Universities Laval. The capabilities of SimPower Systems for modeling a typical electrical system are illustrated in demonstration files. And for users who want to refresh their knowledge of power system theory, there are also self-learning case studies. The SimPower Systems main library, power lib, organizes its blocks into libraries according to their behavior. The powerlib library window displays the block library icons and names. Double-click a library icon to open the library and access the blocks. The main SimPower Systems powerlib library window also contains the Powergui block that opens a graphical user interface for the steady-state analysis of electrical circuits.

Building the Electrical Circuit with powerlib Library:

The graphical user interface makes use of the Simulink functionality to interconnect various electrical components. The electrical components are grouped in a library called powerlib.

1.Open the SimPower Systems library by entering the following command at the MATLAB prompt.

This command displays a Simulink window showing icons of different block libraries.

You can open these libraries to produce the windows containing the blocks  to be copied into your circuit. Each component is represented by a special icon having one or several inputs and outputs corresponding to the different terminals of the component:

2.From the File menu of the powerlib window,open a new window to contain your first circuit and save it as circuit1.

3.Open the Electrical Sources library and copy the AC Voltage Source block into the circuit1 window. C components have disappeared so that the icon now shows a single resistor.

Interfacing the Electrical Circuit with Simulink:

The Voltage Measurement block acts as an interface between the SimPowerSystems blocks and the Simulink blocks. For the system shown above, you implemented such an interface from the electrical system to the Simulink system. The Voltage Measurement block converts the measured voltages into Simulink signals.Similarly, the Current Measurement block from the Measurements library of powerlib can be used to convert any measured current into a Simulink signal.

Measuring Voltages and Currents:

When you measure a current using a Current Measurement block, the positive direction of current is indicated on the block icon (positive current flowing from + terminal to – terminal). Similarly, when you measure a voltage using a Voltage Measurement block, the measured voltage is the voltage of the + terminal with respect to the – terminal. However, when voltages and currents of blocks from the Elements library are measured using the Multimeter block, the voltage and current polarities are not immediately obvious because blocks might have been rotated and there are no signs indicating polarities on the block icons.

Basic Principles of Connecting Capacitors and Inductors

when you connect capacitor elements:

Together with voltage sources, or inductor elements in series with current sources. When you start the simulation, SimPower Systems will signal an error if one of the following two connection errors are present in your diagram:

You have connected a voltage source in parallel with a capacitor, or a series of capacitor elements in series, like in the two examples below .

To fix this problem, you can add a small resistance in series between the voltage source and the capacitors. You have connected a current source in series with an inductor, or a series of inductors connected in parallel, like in the example below. To fix this problem, you can add a large resistance in parallel with the inductor and the capacitors.

Simulation circuit diagram

The simulation circuit diagram of A Dual-Output Wireless Power Transfer System with Active Rectifier and Three-Level Operation.

FORWARD MODE

REVERSE MODE

Chapter-6

Hardware details:

Block diagram:

PIC 16F877A

POWER SUPPLY DESIGN

Driver circuit needs 12V and 5V. Microcontroller need 5V supply, so we convert 230V AC supply is first step down in to 15V by using step down transformer. Then this 15V AC is converted in to DC by using Full bridge rectifier which has high efficiency than all other methods. This 15V DC is converting into 12V DC and 5v DC by using 7812 and 7805 regulator respectively. The capacitor is used to provide smooth variation in voltage. For indication purpose we used LED with 1K resistor to limit current flow to the LED. The following figure shows the regulated power supply.

Regulated power supply

PIC 16F877A:

The microcontrollers played revolutionary role in embedded industry after the invention of Intel 8051. The steady and progressive research in this field gave the industry more efficient, high-performance and low-power consumption microcontrollers. The AVR, PIC and ARM are the prime examples. The new age microcontrollers are getting smarter and richer by including latest communication protocols like USB, I2C, SPI, Ethernet, CAN etc.

PIC microcontrollers

History:

The PIC microcontroller was developed by General Instruments in 1975. PIC was developed when Microelectronics Division of General Instruments was testing its 16-bit CPU CP1600. Although the CP1600 was a good CPU but it had low I/O performance. The PIC controller was used to offload the I/O the tasks from CPU to improve the overall performance of the system.

In 1985, General Instruments converted their Microelectronics Division to Microchip Technology. PIC stands for Peripheral Interface Controller. The General Instruments used the acronyms Programmable Interface Controller and Programmable Intelligent Computer for the initial PICs (PIC1640 and PIC1650).

In 1993, Microchip Technology launched the 8-bit PIC16C84 with EEPROM which could be programmed using serial programming method. The improved version of PIC16C84 with flash memory (PIC18F84 and PIC18F84A) hit the market in 1998.

Development:

Since 1998, Microchip Technology continuously developed new high performance microcontrollers with new complex architecture and enhanced in-built peripherals. PIC microcontroller is based on Harvard architecture. At present PIC microcontrollers are widely used for industrial purpose due to its high performance ability at low power consumption. It is also very famous among hobbyists due to moderate cost and easy availability of its supporting software and hardware tools like compilers, simulators, debuggers etc. The 8-bit PIC microcontroller is divided into following four categories on the basis of internal architecture:

1. Base Line PIC

2. Mid-Range PIC

3. Enhanced Mid-Range PIC

4. PIC18

1. Base Line PIC

Base Line PICs are the least complex PIC microcontrollers. These microcontrollers work on 12-bit instruction architecture which means that the word size of instruction sets are of 12 bits for these controllers. These are smallest and cheapest PICs, available with 6 to 40 pin packaging.  The small size and low cost of Base Line PIC replaced the traditional ICs like 555, logic gates etc. in industries.

2. Mid-Range PIC

Mid-Range PICs are based on 14-bit instruction architecture and are able to work up to 20 MHz speed. These controllers are available with 8 to 64 pin packaging. These microcontrollers are available with different peripherals like ADC, PWM, Op-Amps and different communication protocols like USART, SPI, I2C (TWI), etc. which make them widely usable microcontrollers not only for industry but for hobbyists as well.

Microcontroller PIC16F877A

Microcontroller PIC16F877A is one of the PICMicro Family microcontroller which is popular at this moment, start from beginner until all professionals. Because very easy using PIC16F877A and use FLASH memory technology so that can be write-erase until thousand times. The superiority this Risc Microcontroller compared to with other microcontroller 8-bit especially at a speed of and his code compression. PIC16F877A have 40 pin by 33 path of I/O.

PIC16F877A perfectly fits many uses, from automotive industries and controlling home appliances to industrial instruments, remote sensors, electrical door locks and safety devices. It is also ideal for smart cards as well as for battery supplied devices because of its low consumption. EEPROM memory makes it easier to apply microcontrollers to devices where permanent storage of various parameters is needed (codes for transmitters, motor speed, receiver frequencies, etc.). Low cost, low consumption, easy handling and flexibility make PIC16F877A applicable even in areas where microcontrollers had not previously been considered (example: timer functions, interface replacement in larger systems, coprocessor applications, etc.).In System Programmability of this chip (along with using only two pins in data transfer) makes possible the flexibility of a product, after assembling and testing have been completed. This capability can be used to create assembly-line production, to store calibration data available only after final testing, or it can be used to improve programs on finished products.

High-Performance RISC CPU:

- Only 35 single-word instructions to learn

- All single-cycle instructions except for program branches, which are two-cycle

- Operating speed: DC – 20 MHz clock input DC – 200 ns instruction cycle

- Up to 8K x 14 words of Flash Program Memory, Up to 368 x 8bytes of Data Memory (RAM), Up to 256 x 8 bytes of EEPROM Data Memory

- Pinout compatible to other 28-pin or 40/44-pin PIC16CXXX and PIC16FXXX microcontrollers

Peripheral Features:

- Timer0: 8-bit timer/counter with 8-bit prescaler

- Timer1: 16-bit timer/counter with prescaler, can be incremented during Sleep via external crystal/clock

- Timer2: 8-bit timer/counter with 8-bit period register, prescaler and postscaler

- Two Capture, Compare, PWM modules

- Synchronous Serial Port (SSP) with SPI™ (Master mode) and I2C™ (Master/Slave)

- Universal Synchronous Asynchronous Receiver

- Transmitter (USART/SCI) with 9-bit address detection

- Parallel Slave Port (PSP) – 8 bits wide with external RD, WR and CS controls (40/44-pin only)

- Brown-out detection circuitry for Brown-out Reset (BOR)

Analog Features:

- 10-bit, up to 8-channel Analog-to-Digital Converter (A/D)

- Brown-out Reset (BOR)

- Analog Comparator module (Two analog comparators , Programmable on-chip voltage reference (VREF) module , Programmable input multiplexing from device inputs and internal voltage reference , Comparator outputs are externally accessible)

Special Microcontroller Features:

- 100,000 erase/write cycle Enhanced Flash program memory typical

- 1,000,000 erase/write cycle Data EEPROM memory typical

- Data EEPROM Retention > 40 years

- Self-reprogrammable under software control

- In-Circuit Serial Programming™ (ICSP™) via two pins

- Single-supply 5V In-Circuit Serial Programming

- Watchdog Timer (WDT) with its own on-chip RC oscillator for reliable operation

- Programmable code protection

- Power saving Sleep mode

- Selectable oscillator options

- In-Circuit Debug (ICD) via two pins

CMOS Technology:

- Low-power, high-speed Flash/EEPROM technology

- Fully static design

- Wide operating voltage range (2.0V to 5.5V)

- Commercial and Industrial temperature ranges

- Low-power consumption

PIN DIAGRAM:

PIN DESCRIPTION:

- OSC1/CLKI - Oscillator crystal or external clock input.Oscillator crystal input or external clock source input. ST buffer when configured in RC mode;otherwise CMOS.External clock source input. Always associated with pin function OSC1 (see OSC1/CLKI,OSC2/CLKO pins).

- OSC2/CLKO - Oscillator crystal or clock output.Oscillator crystal output.

Connects to crystal or resonator in Crystal Oscillator mode.In RC mode, OSC2 pin outputs CLKO, which has 1/4 the frequency of OSC1 and denotes the instruction cycle rate.

- MCLR/VPP - Master Clear (input) or programming voltage (output).Master Clear (Reset) input. This pin is an active low Reset to the device.Programming voltage input.

- RA0/AN0 - PORTA is a bidirectional I/O port. Digital I/O.

Analog input 0.

- RA1/AN1 - Digital I/O. Analog input 0.

- RA2/AN2/VREF-/CVREF - Digital I/O.Analog input 2.A/D reference voltage (Low) input.Comparator VREF output.

- RA3/AN3/VREF+ -  Digital I/O.Analog input 3.A/D reference voltage (High) input.

- RA4/T0CKI/C1OUT - Digital I/O – Open-drain when configured as output.Timer0 external clock input.Comparator 1 output.

- RA5/AN4/SS/C2OUT - Digital I/O.Analog input 4.SPI slave select input.Comparator 2 output.

- RB0/INT - PORTB is a bidirectional I/O port. PORTB can be software programmed for internal weak pull-up on all inputs.Digital I/O. External interrupt.

- RB1 - Digital I/O.

- RB2 - Digital I/O.

- RB3/PGM - Digital I/O.Low-voltage ICSP programming enable pin.

- RB4 - Digital I/O.

- RB5 - Digital I/O.

- RB6/PGC - Digital I/O.In-circuit debugger and ICSP programming clock.

- RB7/PGD - Digital I/O.In-circuit debugger and ICSP programming data.

PIC16F874A/877A BLOCK DIAGRAM

MEMORY ORGANIZATION

There are three memory blocks in each of the PIC16F87XA devices. The program memory and data memory have separate buses so that concurrent access can occur and is detailed in this section. The EEPROM data memory block is detailed in Section 3.0 “Data EEPROM and Flash Program Memory”.

Program Memory Organization

The PIC16F87XA devices have a 13-bit program counter capable of addressing an 8K word x 14 bit program memory space. The PIC16F876A/877A devices have 8K words x 14 bits of Flash program memory, while PIC16F873A/874A devices have 4K words x 14 bits. Accessing a location above the physically implemented address will cause a wraparound. The Reset vector is at 0000h and the interrupt vector isat 0004h.

FIGURE 2-1:     PIC16F876A/877A

PROGRAM MEMORY MAP

AND STACK

Data Memory Organization

The data memory is partitioned into multiple banks which contain the General Purpose Registers and the Special Function Registers. Bits RP1 (Status<6>) and RP0 (Status<5>) are the bank select bits.

Each bank extends up to 7Fh (128 bytes). The lower locations of each bank are reserved for the Special Function Registers. Above the Special Function Registers are General Purpose Registers, implemented as static RAM. All implemented banks contain Special Function Registers. Some frequently used Special Function Registers from one bank may be mirrored in another bank for code reduction and quicker access.

GENERAL PURPOSE REGISTER FILE

The register file can be accessed either directly, or indirectly, through the File Select Register (FSR).

SPECIAL FUNCTION REGISTERS

The Special Function Registers are registers used by the CPU and peripheral modules for controlling the desired operation of the device. These registers are implemented as static RAM. The Special Function Registers can be classified into two sets: core (CPU) and peripheral.

Status Register

The Status register contains the arithmetic status of the ALU, the Reset status and the bank select bits for data memory. The Status register can be the destination for any instruction, as with any other register. If the Status register is the destination for an instruction that affects the Z, DC or C bits, then the write to these three bits is disabled. These bits are set or cleared according to the device logic. Furthermore, the TO and PD bits are not writable, therefore, the result of an instruction with the Status register as destination may be different than intended.

For example, CLRF STATUS, will clear the upper three bits and set the Z bit. This leaves the Status register as 000u u1uu (where u = unchanged).

It is recommended, therefore, that only BCF, BSF, SWAPF and MOVWF instructions are used to alter the Status register because these instructions do not affect the Z, C or DC bits from the Status register.

PCL and PCLATH

The Program Counter (PC) is 13 bits wide. The low byte comes from the PCL register which is a readable and writable register. The upper bits (PC<12:8>) are not readable, but are indirectly writable through the PCLATH register. On any Reset, the upper bits of the PC will be cleared. Figure 2-5 shows the two situations for the loading of the PC. The upper example in the figure shows how the PC is loaded on a write to PCL (PCLATH<4:0> → PCH). The lower example in the figure shows how the PC is loaded during a CALL or GOTO instruction (PCLATH<4:3> → PCH).

FIGURE 2-5: LOADING OF PC IN

DIFFERENT SITUATIONS

COMPUTED GOTO

A computed GOTO is accomplished by adding an offset to the program counter (ADDWF PCL). When doing a table read using a computed GOTO method, care should be exercised if the table location crosses a PCL memory boundary (each 256-byte block). Refer to the application note, AN556, “Implementing a Table Read” (DS00556).

STACK

The PIC16F87XA family has an 8-level deep x 13-bit wide hardware stack. The stack space is not part of either program or data space and the stack pointer is not readable or writable. The PC is PUSHed onto the stack

when a CALL instruction is executed, or an interrupt causes a branch. The stack is POP’ed in the event of a RETURN, RETLW or a RETFIE instruction execution. PCLATH is not affected by a PUSH or POP operation.

The stack operates as a circular buffer. This means that after the stack has been PUSHed eight times, the ninth push overwrites the value that was stored from the first push. The tenth push overwrites the second push (and

so on).

Program Memory Paging

All PIC16F87XA devices are capable of addressing a continuous 8K word block of program memory. The CALL and GOTO instructions provide only 11 bits of address to allow branching within any 2K program memory page. When doing a CALL or GOTO instruction, the upper 2 bits of the address are provided by PCLATH<4:3>. When doing a CALL or GOTO instruction,

the user must ensure that the page select bits are programmed so that the desired program memory page is addressed. If a return from a CALL instruction (or interrupt) is executed, the entire 13-bit PC is popped

off the stack. Therefore, manipulation of the PCLATH<4:3> bits is not required for the RETURN instructions (which POPs the address from the stack).

DATA EEPROM AND FLASH PROGRAM MEMORY

The data EEPROM and Flash program memory is readable and writable during normal operation (over the full VDD range). This memory is not directly mapped in the register file space. Instead, it is indirectly addressed

through the Special Function Registers. There are six SFRs used to read and write this memory:

• EECON1

• EECON2

• EEDATA

• EEDATH

• EEADR

• EEADRH

When interfacing to the data memory block, EEDATA holds the 8-bit data for read/write and EEADR holds the address of the EEPROM location being accessed. These devices have 128 or 256 bytes of data EEPROM (depending on the device), with an address range from 00h to FFh. On devices with 128 bytes, addresses from 80h to FFh are unimplemented and will wraparound to

the beginning of data EEPROM memory. When writing to unimplemented locations, the on-chip charge pump will be turned off.

When interfacing the program memory block, the EEDATA and EEDATH registers form a two-byte word that holds the 14-bit data for read/write and the EEADR and EEADRH registers form a two-byte word that holds the 13-bit address of the program memory location being accessed. These devices have 4 or 8K words of program Flash, with an address range from 0000h to 0FFFh for the PIC16F873A/874A and 0000h to 1FFFh for the PIC16F876A/877A. Addresses above the range of the respective device will wraparound to the beginning of program memory.

The EEPROM data memory allows single-byte read and write. The Flash program memory allows single-word reads and four-word block writes. Program memory write operations automatically perform an erase-before write on blocks of four words. A byte write in data EEPROM memory automatically erases the location and writes the new data (erase-before-write).

The write time is controlled by an on-chip timer. The write/erase voltages are generated by an on-chip charge pump, rated to operate over the voltage range of the device for byte or word operations.

When the device is code-protected, the CPU may continue to read and write the data EEPROM memory. Depending on the settings of the write-protect bits, the device may or may not be able to write certain blocks

of the program memory; however, reads of the program memory are allowed. When code-protected, the device programmer can no longer access data or program memory; this does NOT inhibit internal reads or writes.

EEADR and EEADRH

The EEADRH: EEADR register pair can address up to a maximum of 256 bytes of data EEPROM or up to a maximum of 8K words of program EEPROM. When selecting a data address value, only the LSByte of the address is written to the EEADR register. When selecting a program address value, the MSByte of the address is written to the EEADRH register and the LSByte is written to the EEADR register. If the device contains less memory than the full address reach of the address register pair, the Most Significant bits of the registers are not implemented. For example, if the device has 128 bytes of data EEPROM, the Most Significant bit of EEADR is not implemented on access to data EEPROM.

EECON1 and EECON2 Registers

EECON1 is the control register for memory accesses. Control bit, EEPGD, determines if the access will be a program or data memory access. When clear, as it is when reset, any subsequent operations will operate on the data memory. When set, any subsequent operations will operate on the program memory. Control bits, RD and WR, initiate read and write or erase, respectively. These bits cannot be cleared, only set, in software. They are cleared in hardware at completion of the read or write operation. The inability to clear the WR bit in software prevents the accidental, premature termination of a write operation.

The WREN bit, when set, will allow a write or erase operation. On power-up, the WREN bit is clear. The WRERR bit is set when a write (or erase) operation is interrupted by a MCLR or a WDT Time-out Reset during normal operation. In these situations, following Reset, the user can check the WRERR bit and rewrite the location. The data and address will be unchanged in the EEDATA and EEADR registers.

Interrupt flag bit, EEIF in the PIR2 register, is set when the write is complete. It must be cleared in software. EECON2 is not a physical register. Reading EECON2 will read all ‘0’s. The EECON2 register is used exclusively in the EEPROM write sequence.

Buffer:

Voltage buffer

A voltage buffer amplifier is used to transfer a voltage from a first circuit, having a high output impedance level, to a second circuit with a low input impedance level. The interposed buffer amplifier prevents the second circuit from loading the first circuit unacceptably and interfering with its desired operation. In the ideal voltage buffer in the diagram, the input resistance is infinite, the output resistance zero (impedance of an ideal voltage source is zero). Other properties of the ideal buffer are: perfect linearity, regardless of signal amplitudes; and instant output response, regardless of the speed of the input signal.

If the voltage is transferred unchanged (the voltage gain Av is 1), the amplifier is a unity gain buffer; also known as a voltage follower because the output voltage follows or tracks the input voltage. Although the voltage gain of a voltage buffer amplifier may be (approximately) unity, it usually provides considerable current gain and thus power gain. However, it is commonplace to say that it has a gain of 1 (or the equivalent 0 dB), referring to the voltage gain.

As an example, consider a Thévenin source (voltage VA, series resistance RA) driving a resistor load RL. Because of voltage division (also referred to as "loading") the voltage across the load is only VA RL / ( RL + RA ). However, if the Thévenin source drives a unity gain buffer such as that in Figure 1 (top, with unity gain), the voltage input to the amplifier is VA, and with no voltage division because the amplifier input resistance is infinite. At the output the dependent voltage source delivers voltage Av VA = VA to the load, again without voltage division because the output resistance of the buffer is zero. A Thévenin equivalent circuit of the combined original Thévenin source and the buffer is an ideal voltage source VA with zero Thévenin resistance.

Typically a current buffer amplifier is used to transfer a current from a first circuit, having a low output impedance level, to a second circuit with a high input impedance level. The interposed buffer amplifier prevents the second circuit from loading the first circuit unacceptably and interfering with its desired operation. In the ideal current buffer in the diagram, the input impedance is zero and the output impedance is infinite (impedance of an ideal current source is infinite). Again, other properties of the ideal buffer are: perfect linearity, regardless of signal amplitudes; and instant output response, regardless of the speed of the input signal.

For a current buffer, if the current is transferred unchanged (the current gain βi is 1), the amplifier is again a unity gain buffer; this time known as a current follower because the output current follows or tracks the input current.

As an example, consider a Norton source (current IA, parallel resistance RA) driving a resistor load RL. Because of current division (also referred to as "loading") the current delivered to the load is only IA RA / ( RL + RA ). However, if the Norton source drives a unity gain buffer such as that in Figure 1 (bottom, with unity gain), the current input to the amplifier is IA, with no current division because the amplifier input resistance is zero. At the output the dependent current source delivers current βi IA = IA to the load, again without current division because the output resistance of the buffer is infinite. A Norton equivalent circuit of the combined original Norton source and the buffer is an ideal current source IAwith infinite Norton resistance.

Figure 1: Top: Ideal voltage buffer Bottom: Ideal current buffer

Current buffer

Typically a current buffer amplifier is used to transfer a current from a first circuit, having a low output impedance level, to a second circuit with a high input impedance level. The interposed buffer amplifier prevents the second circuit from loading the first circuit unacceptably and interfering with its desired operation. In the ideal current buffer in the diagram, the input impedance is zero and the output impedance is infinite (impedance of an ideal current source is infinite). Again, other properties of the ideal buffer are: perfect linearity, regardless of signal amplitudes; and instant output response, regardless of the speed of the input signal.

For a current buffer, if the current is transferred unchanged (the current gain βi is 1), the amplifier is again a unity gain buffer; this time known as a current follower because the output current follows or tracks the input current.

As an example, consider a Norton source (current IA, parallel resistance RA) driving a resistor load RL. Because of current division (also referred to as "loading") the current delivered to the load is only IA RA / ( RL + RA ). However, if the Norton source drives a unity gain buffer such as that in Figure 1 (bottom, with unity gain), the current input to the amplifier is IA, with no current division because the amplifier input resistance is zero. At the output the dependent current source delivers current βi IA = IA to the load, again without current division because the output resistance of the buffer is infinite. A Norton equivalent circuit of the combined original Norton source and the buffer is an ideal current source IAwith infinite Norton resistance.

These buffers/line drivers are designed to improve both the performance and PC board density of TRI-STATE buffers/drivers employed as memory-address Drivers, clock drivers, and bus-oriented transmitters/receivers. Featuring 400 Mv of hysteresis at each low current PNP data line input, they provide improved noise rejection and high fan out outputs and can be used to drive terminated lines down to 133X.

3.5.1FEATURES:

- TRI-STATE outputs drive bus lines directly.

- PNP inputs reduce DC loading on bus lines.

- Hysteresis at data inputs improves noise margins.

Typical IOL (sink current)                   74LS 64 mA

Typical IOH (source current)               74LS b15 mA

Typical propagation delay times          Inverting 10.5ns Non inverting16ns

Typical enable/disable time 		       18 ns

Inverting                                                  130 mW

Non inverting                                           135 mW

3.5.2 CONNECTION DIAGRAM :

Fig.3.5 Connection Diagram of Buffer Circuit.

TOP VIEW FOR 74LS244

3.5.3 FUNCTION TABLE

POWER MOSFET BASICS

The MOSFET, or Metal-Oxide-Semiconductor, Field-Effect Transistor is by far the most common field effect transistor in both digital and analog circuits. The MOSFET is composed of a channel of n-type or p-type semiconductor material, and is accordingly called an NMOSFET or a PMOSFET. Unfortunately, many semiconductors with better electrical properties than silicon, such as gallium arsenide, do not form good gate oxides and thus are not suitable for MOSFETs.

The gate terminal is a layer of polysilicon (polycrystalline silicon) or aluminum placed over the channel, but separated from the channel by a thin layer of insulating silicon dioxide.

FEATURES OF POWER MOSFET’S

- Power MOSFET has lower switching losses but its on-resistance and conduction losses are more.

- MOSFET is a voltage-controlled device.

- MOSFET has positive temperature co-efficient for resistance. This makes parallel operation of MOSFET easy. If a MOSFET shares increased current initially, it heats up faster its resistance rises and this increased resistance causes this current to shift to other devices in parallel

- In MOSFET secondary break down does not occur, because it has positive temperature co-efficient.

- Power MOSFET’s in higher voltage ratings have more conduction losses

- The state of the art MOSFET’s are available with ratings up to 500V, 140A.

PROPERTIES AND CHARACTERISTICS OF MOSFET

The transfer characteristic of an N-channel MOSFET is shown in fig below

Fig 3.1 Transfer characteristic of N-channel MOSFET

Threshold voltage Vt6 is defined as the minimum gate electrode bias required strongly inverting the surface under the poly and forming a conducting channel between the source and the drain regions. Vt is usually measured at a drain-source current of 650mA. Common values are 6-4V for high voltage devices with thicker gate oxides, and 1-6V for lower voltage, logic-compatible devices with thinner gate oxides. Generally the gate-source voltage is chosen somewhere in the linear region of the transfer characteristics.

The MOSFET has three modes of operation, one of which that acts like a switch that is off, and two for which the MOSFET acts as a switch that is on. For the NMOSFET the modes are

1. Cut-off: When VGS<Vth where Vth is the threshold voltage of the device.  Here the switch is turned off, and there is no conduction between drain and source. While the current between drain and source should ideally be 0 since the switch is turned off, there is a weak-inversion current or sub threshold leakage.

2. Triode: When VGS>Vth and VDS<VGS-Vth, the switch is turned on, and a channel has been created which allows current to flow between the drain and source. The MOSFET operates like a resistor.

3. Saturation: When VGS>Vth and VDS>VGS-Vth, the switch is turned on, and a channel has been created which allows current to flow between the drain and source, but the current is not a function of the voltage difference in the channel, and thus the MOSFET does not operate as a resistor, but instead it operates as an amplifier.

N-CHANNEL TRENCHMOS TRANSISTOR (IRF840)

FEATURES

- Trench technology.

- Low ON-state resistance.

- Fast switching.

- Low thermal resistance.

APPLICATIONS

- DC to DC converters.

- Switched mode converters.

- T.V and computer monitor power supplies.

The IRF840 is supplied in the SOT78(TO660AB) convention leaded package

The IRF840 is supplied in the SOT404 (D6PAK) surface-mounting package.

3.3.3 Description:

The IRF-840 provides fast switching, ruggedized device design, low on-resistance and cost effectiveness.

The TO-660 package is universally preferred for all commercial-industrial applications at power dissipation levels to approximately   50 watts. The low thermal resistance and low package cost of the   TO-660 contribute to its wide acceptance throughout the industry Gate Drive Characteristics and Requirements for HEXFETs

### Gate drive vs base drive

IA. Enhancement vs Depletion

IB. N vs P-Channel

The conventional bipolar transistor is a current-driven device. As illustrated in Figure 1(a). a current must be  applied between the base and emitter terminals to produce a flow of current in the collector. The amount of  a drive required to produce a given output depends upon the gain, but invariably a current must be made to flow into the base terminal to produce a flow of current in the collector.

Figure 3.2. Bipolar transistor is current driven, HEXFET is voltage driven.

The HEXFET is fundamentally different: it is a voltage-controlled power MOSFET device. A voltage must be applied between the gate and source terminals to produce a flow of current in the drain (see Figure 1b). The gate is isolated electrically from the source by a layer of silicon dioxide. Theoretically, therefore, no current flows into the gate when a DC voltage is applied to it - though in practice there will be an extremely small current, in the order of nanoamperes. With no voltage applied between the gate and source electrodes, the impedance between the drain and source terminals is very high, and only the leakage current flows in the drain.

11k 
Figure 3.3. Basic HEXFET structure

When a voltage is applied between the gate and source terminals, an electric field is set up within the HEXFET.

This field "inverts" the channel (Figure 6) from P to N, so that a current can flow from drain to source in an uninterrupted sequence of N-type silicon (drain-channel-source).

Enhancement vs Depletion

Field-effect transistors can be of two types: enhancement mode and depletion mode. Enhancement-mode devices need a gate voltage of the same sign as the drain voltage in order to pass current. Depletion-mode devices are naturally on and are turned off by a gate voltage of the same polarity as the drain voltage. All HEXFETs are enhancement-mode devices.

N vs P-Channel

All MOSFET voltages are referenced to the source terminal. An N-Channel device, like an NPN transistor, has a drain voltage that is positive with respect to the source. Being enhancement-mode devices, they will be turned on by a positive voltage on the gate. The opposite is true for P-Channel devices, that are similar to PNP transistors.

Although it is common knowledge that HEXFET transistors are more easily driven than bipolar, a few basic considerations have to be kept in mind in order to avoid a loss in performance or outright device failure.

3.4.1FEATURES

- Ultra Low On-Resistance

- Dynamic dv/dt Rating

- 175°C Operating Temperature

- Fast Switching

- Fully Avalanche Rated

- 8A, 500V

- rDS(ON) = 0.850W

- Single Pulse Avalanche Energy Rated

- SOA is Power Dissipation Limited

- Nanosecond Switching Speeds

SYMBOL FOR POWER MOSFET

Fig 3.4 Power MOSFET

# ISOLATION CIRCUIT

Isolation circuits are specially designed circuits to isolate the POWER CIRCUIT and CONTROLLER CIRCUIT. These circuits are used to provide ground. ICs are usually used to provide this isolation.

Gate Driver  :

A gate driver is a power amplifier that accepts a low-power input from a controller IC and produces a high-current drive input for the gate of a high-power transistor such as an IGBT or power MOSFET. Gate drivers can be provided either on-chip or as a discrete module. In essence, a gate driver consists of a level shifter in combination with an amplifier.

Purpose :

It is often stated that transistors such as MOSFETs with isolated gate electrodes can be driven without a power source, which is not correct. In contrast to bipolar transistors, MOSFETs do not require constant power input, as long as they are not being switched on or off. The isolated gate-electrode of the MOSFET forms a capacitor (gate capacitor), which must be charged or discharged each time the MOSFET is switched on or off. As a transistor requires a particular gate voltage in order to switch on, the gate capacitor must be charged to at least the required gate voltage for the transistor to be switched on. Similarly, to switch the transistor off, this charge must be dissipated, i.e. the gate capacitor must be discharged.

When a transistor is switched on or off, it does not immediately switch from a non-conducting to a conducting state; and may transiently support both a high voltage and conduct a high current. Consequently, when gate current is applied to a transistor to cause it to switch, a certain amount of heat is generated which can, in some cases, be enough to destroy the transistor. Therefore, it is necessary to keep the switching time as short as possible, so as to minimize switching loss. Typical switching times are in the range of microseconds. The switching time of a transistor is inversely proportional to the amount of current used to charge the gate. Therefore, switching currents are often required in the range of several hundred milli amperes, or even in the range of amperes. For typical gate voltages of approximately 10-15V, several watts of power may be required to drive the switch. When large currents are switched at high frequencies, (e.g. in DC-to-DC converters of large electric motors), multiple transistors are sometimes provided in parallel, so as to provide sufficiently high switching currents and switching power.

The switching signal for a transistor is usually generated by a logic circuit or a microcontroller, which provides an output signal that typically is limited to a few milli amperes of current. Consequently, a transistor which is directly driven by such a signal would switch very slowly, with correspondingly high power loss. During switching, the gate capacitor of the transistor may draw current so quickly that it causes a current overdraw in the logic circuit or microcontroller, causing overheating which leads to permanent damage or even complete destruction of the chip. To prevent this from happening, a gate driver is provided between the microcontroller output signal and the power transistor.

It is a power amplifier that accepts a low-power input from a controller IC and produces the appropriate high-current gate drive for a power MOSFET. A gate driver is used when a pulse-width-modulation (PWM) controller cannot provide the output current required to drive the gate capacitance of the associated MOSFET. Gate drivers may be implemented as dedicated ICs, discrete transistors, or transformers. They can also be integrated within a controller IC. Partitioning the gate-drive function off the PWM controller allows the controller to run cooler and be more stable by eliminating the high peak currents and heat dissipation needed to drive a power MOSFET at very high frequencies.

Figure 2 shows the typical gate-drive circuit. Generating a gate voltage for the low-side switch is straightforward, as it is ground referenced. But the high-side switch presents a problem. The gate-control voltage, which goes from rail-to-rail, must be referenced to the MOSFET's source terminal, which in this case is not at ground.

Figure 2: Gate drive

The FAN7392 is a monolithic high- and low-side gate drive IC, that can drive high-speed MOSFETs and IGBTs that operate up to +600V. It has a buffered output stage with all NMOS transistors designed for high pulse current driving capability and minimum cross-conduction. Fairchild’s high-voltage process and common-mode noise canceling techniques provide stable operation of the high-side driver under high dv/dt noise circumstances. An advanced level-shift circuit offers high-side gate driver operation up to VS=-9.8V (typical) for VBS=15V. Logic inputs are compatible with standard CMOS or LSTTL output, down to 3.3V logic. The UVLO circuit prevents malfunction when VCC and VBS are lower than the specified threshold voltage. The high-current and low-output voltage drop feature makes this device suitable for halfand full-bridge inverters, like switching-mode power supply and high-power DC-DC converter applications.

FIG.FAN7392

Pin specification:

VDD-Logic supply

HIN-Logic input for high side gate driver output (HO), in phase

SD- Logic input for shutdown

LIN-Logic input for low side gate driver output (LO),in phase

VSS-Logic ground

HO-High side gate driver output

VB-High side floating supply

VS- High side floating supply return

VCC-Low side supply

COM-Low side return

LO-Low side gate driver output

TOP VIEW OF FAN7392:

# Fig.3.10 Typical connection of Gate driver

Lead Definitions:

Features

- Floating Channels for Bootstrap Operation to +600V

- 3A/3A Sourcing/Sinking Current Driving Capability

- Common-Mode dv/dt Noise Canceling Circuit

- 3.3V Logic Compatible

- Separate Logic Supply (VDD) Range from 3.3V to 20V

- Under-Voltage Lockout for VCC and VBS

- Cycle-by-Cycle Edge-Triggered Shutdown Logic

- Matched Propagation Delay for Both Channels

- Outputs In-phase with Input Signals

- Available in 14-DIP and 16-SOP (Wide) Packages

Functional Blockdiagram :

Pin Configuration:

VCC is the low-side supply and should be between 10V and 20V. VDD is the logic supply to the FAN7392. It can be between +3V to +20V (with reference to VSS). The actual voltage you choose to use depends on the voltage level of your input signals. Here’s the chart:

Fig. 4 - FAN7392 Logic "1" Input Threshold vs VDD

It is common practice to use VDD = +5V. When VDD = +5V, the logic 1 input threshold is slightly higher than 3V. Thus when VDD = +5V, the FAN7392 can be used to drive loads when input “1” is higher than 3 point something volts. This means that it can be used for almost all circuits, since most circuits tend to have around 5V outputs. When you’re using microcontrollers the output voltage will be higher than 4V (when the microcontroller has VDD = +5V, which is quite common). When you’re using SG3525 or TL494 or other PWM controller, you are probably going to have them powered off greater than 10V, meaning the outputs will be higher than 8V when high. So, the FAN7392 can be easily used.

You may lower the VDD down to about 4V if you’re using a microcontroller or any chip that gives output of 3.3V . While designing circuits with the FAN7392, I had noticed that sometimes the circuit didn’t work properly when FAN7392 VDD was selected as less than +4V. So, I do not recommend using VDD less than +4V.

In most of my circuits, I do not have signal levels which have voltages less than 4V as high and so I use VDD = +5V.

If for some reason, you have signals levels with logic “1” having lower than 3V, you will need a level converter / translator that will boost the voltage to acceptable limits. In such situations, I recommend boosting up to 4V or 5V and using FAN7392 VDD = +5V.

VSS is the logic supply ground. COM is “low side return” – basically, low side drive ground connection. It seems that they are independent and you might think you could perhaps isolate the drive outputs and drive signals. However, you’d be wrong. While they are not internally connected, FAN7392 is a non-isolated driver, meaning that VSS and COM should both be connected to ground.

HIN and LIN are the logic inputs. A high signal to HIN means that you want to drive the high-side MOSFET, meaning a high output is provided on HO. A low signal to HIN means that you want to turn off the high-side MOSFET, meaning a low output is provided on HO. The output to HO – high or low – is not with respect to ground, but with respect to VS. We will soon see how a bootstrap circuitry (diode + capacitor) – utilizing VCC, VB and VS – is used to provide the floating supply to drive the MOSFET. VS is the high side floating supply return. When high, the level on HO is equal to the level on VB, with respect to VS. When low, the level on HO is equal to VS, with respect to VS, effectively zero.

A high signal to LIN means that you want to drive the low-side MOSFET, meaning a high output is provided on LO. A low signal to LIN means that you want to turn off the low-side MOSFET, meaning a low output is provided on LO. The output on LO is with respect to ground. When high, the level on LO is equal to the level of VCC, with respect to VSS, effectively ground. When low, the level on LO is equal to the level on VSS, with respect to VSS, effectively zero.

SD is used as shutdown control. When this pin is low, FAN7392 is enabled – shutdown function is disabled. When this pin is high, the outputs are turned off, disabling the FAN7392 drive.

REFERENCES

[1] V. Valente, et al., “A high-power CMOS class-D amplifier for inductive link medical transmitters,” IEEE Trans. Power Electron., vol. 30, no. 8,pp. 4477–4488, Aug. 2015.

[2] Y. Lu, et al., “A 13.56 MHz fully integrated 1X/2X active rectifier with compensated bias current for inductively powered devices,” in Proc. IEEE ISSCC Dig. Tech. Papers, Feb. 2013, pp. 66–67.

[3] J.-H. Choi, et al., “Resonant regulating rectifiers (3R) operating for 6.78 MHz resonant wireless power transfer (RWPT),” IEEE J. Solid-State Circuits, vol. 48, no. 12, pp. 2989–3001, Dec. 2013.

[4] H. Park, et al., “A design of a wireless power receiving unit with a highefficiency 6.78-MHz active rectifier using shared DLLs for magneticresonant A4 WP applications,” IEEE Trans. Power Electron., vol. 31, no. 6, pp. 4484–4498, Jun. 2016.